ALGORITHM (FUZZY CONTROLLER) DOCUMENTATION GUIDE

Provide the required details using the following guide.

By John Lavack 20001249

| Components | Details | Intructions |
| --- | --- | --- |
| Inputs |  | Specify all the inputs, including the coefficients (constants). Note: This assignment requires implementing Yamakawa’s fuzzy controller design. |
| Fuzzy Rules | Two FAMMS: Inputs in column and row titles. Outputs inside matrix NL; NS; ZE; PS; PL. NL; NVL; NL; NL; NM; NM. NS; NL; NM; NS; NS; PS. ZE; NM; NS; ZE; PS; PM. PS; NS; PS; PS; PM; PL. PL; PM; PM; PL; PL; PVL. NL; NS; ZE; PS; PL. NL; PVL; PL; PM; PS; ZE. NS; PL; PM; PS; ZE; NS. ZE; PM; PS; ZE; NS; NM. PS; PS; ZE; NZ; NM; NL. PL; ZE; NS; NM; NL; NVL. NVL = -400 NL = -200 NM = -100 NS = -50 ZE = 0 PS = 50 PM = 100 PL = 200 PVL = 400 | Specify all the fuzzy rules in the system. |
| Fuzzy Membership functions | notation: input ## = type {a,b,c,d} For X: input nl = left { -1.75, -1.25, 0, 0} input ns = regular { -1.75, -1.25, -0.75, -0.25} input ze =regular { -0.75, -0.25, 0.25, 0.75} input ps = regular{ 0.25, 0.75, 1.25, 1.75} input pl = right { 1.25, 1.75, 0, 0} For Y: input nl = left { -1.75, -1.25, 0, 0} input ns = regular { -1.75, -1.25, -0.75, -0.25} input ze =regular { -0.75, -0.25, 0.25, 0.75} input ps = regular{ 0.25, 0.75, 1.25, 1.75} input pl = right { 1.25, 1.75, 0, 0} | Specify all the parameters of all membership functions used for the all inputs. (e.g. input, type, name, a=?,b=?,c=?,d=?) Use the Fuzzy Sets Viewer Excel file to generate a diagram. |
| Defuzzification Method | Centroid | Specify method used. |